REAGEN™

Thiamphenicol ELISA Test Kit

RND99096

### Product Description

REAGEN™ Thiamphenicol ELISA Test Kit is a competitive enzyme immunoassay for the quantitative analysis of Thiamphenicol in , fish, eggs, meat (beef, chicken and pork), Milk/Condensed milk, milk powder.

The unique features of the kit are:

- High recovery (70-115%), rapid (10 - 40 minutes), and cost-effective extraction methods.

- High sensitivity (0.1 ng/g or ppb) and low detection limit (0.05 ng/g or ppb) for shrimp, fish and meat samples.

- High reproducibility.

- A quick ELISA assay (less than 2 hours regardless of number of samples).

### Procedure Overview

The method is based on a competitive colorimetric ELISA assay. The Thiamphenicol antibody has been coated in the plate wells. During the analysis, sample is added along with the Thiamphenicol-horseradish peroxidase conjugate. If the Thiamphenicol residue is present in the sample, it will compete for the Thiamphenicol antibody, thereby preventing the Thiamphenicol-HRP from binding to the antibody attached to the well. The resulting color intensity, after addition of the HRP substrate (TMB), has an inverse relationship with the Thiamphenicol residue concentration in the sample.

### Kit Contents, Storage and Shelf Life

REAGEN™ Thiamphenicol ELISA Test Kit has the capacity for 96 determinations or testing of 42 samples in duplicate (assuming 12 wells for standards). Return any unused microwells to the foil bag and reseal them with the desiccant provided in the original package. Store the kit at 2-8C *. The shelf life is 12 months when the kit is properly stored.

| Kit Contents | Amount | Storage |
| --- | --- | --- |
| Thiamphenicol Antibody-Coated Plate | 1x96-well Plate (8wells x12strips) | 2-8C |
| Thiamphenicol Standards: Negative control (white cap tube) 0.1 ng/mL (yellow cap tube) 0.3 ng/mL (orange cap tube) 1.0 ng/mL (pink cap tube) 3.0 ng/mL (purple cap tube) 9.0 ng/mL (blue cap tube) 20ng/mL(spiking, optional, red cap tube) | 2.0 mL 2.0 mL 2.0 mL 2.0 mL 2.0 mL 2.0 mL 2.0 mL | 2-8C |
| Thiamphenicol-HRP Conjugate | 8 mL | 2-8C * |
| 10X Sample Extraction Buffer ** | 25 mL | 2-8C |
| 20X Wash Solution ** | 30 mL | 2-8C |
| Stop Buffer ** | 20 mL | 2-8C |
| TMB Substrate ** | 10 mL | 2-8C |

* 	If you are not planning to use the kit for over 3 months, store Thiamphenicol-HRP Conjugate at -20C or in a freezer.

### Sensitivity

| Sample Type | Detection Limit (ng/g or ppb) |
| --- | --- |
| Fish/Shrimp/Meat | 0.05 |
| Egg | 0.1 |
| Milk/Condensed milk | 0.5 |
| Milk powder | 0.2 |

### Specificity

| Analytes | Cross-Reactivity (%) |
| --- | --- |
| Thiamphenicol | 100 |
| Chloramphenicol Glucuronide | < 0.01 |
| Chloramphenicol Base | < 0.01 |
| Chloramphenicol)CAP | < 0.01 |
| Tetracyclines | < 0.01 |
| Gentamicin | < 0.01 |
| Ampicillin | < 0.01 |
| Florfenicol | < 0.01 |

### Required Materials Not Provided With the Kit

- Microtiter plate reader (450 nm)

- Incubator

- Tissue Mixer (e.g. Omni Tissue Master Homogenizer)

- Rotary evaporator or nitrogen gas

- Vortex mixer (e.g. Gneie Vortex mixer from VWR)

- 10, 20, 100 and 1000 L pipettes

- Multi-channel pipette: 50-300 L (Optional)

- Ethyl acetate

- n-Hexane

### Warnings and Precautions

- The standards contain Thiamphenicol. Handle with particular care.

- Do not use the kit past the expiration date.

- Do not intermix reagents from different kits or lots except for components with the same part No’s within their expiration dates. Thiamphenicol-HRP CONJUGATES AND PLATES ARE KIT-AND LOT-SPECIFIC.

- Try to maintain a laboratory temperature of 20°–25°C (68°–77°F). Avoid running assays under or near air vents, as this may cause excessive cooling, heating and/or evaporation. Also, do not run assays in direct sunlight, as this may cause excessive heat and evaporation. Cold bench tops should be avoided by placing several layers of paper towel or some other insulation material under the assay plates during incubation.

- Make sure you are using only distilled or deionized water since water quality is very important.

- When pipetting samples or reagents into an empty microtiter plate, place the pipette tips in the lower corner of the well, making contact with the plastic.

- Incubations of assay plates should be timed as precisely as possible. Be consistent when adding standards to the assay plate. Add your standards first and then your samples.

- Add standards to plate only in the order from low concentration to high concentration as this will minimize the risk of compromising the standard curve.

- Always refrigerate plates in sealed bags with a desiccant to maintain stability. Prevent condensation from forming on plates by allowing them equilibrate to room temperature (20 – 25C / 68 – 77F) while in the packaging.

SAMPLE PREPARATION

Be sure samples are properly stored. In general, samples should be refrigerated at 2-4°C for no more than 1-2 days. Freeze samples to a minimum of -20°C if they need to be stored for a longer period. Frozen samples can be thawed at room temps (20 – 25C / 68 – 77F) or in a refrigerator before use.

- Preparation of 1X Sample Extraction Buffer:

Mix 1 volume of 10X Sample Extraction Buffer with 9 volumes of distilled water.

### Condensed milk

    - Homogenize a reasonable amount of sample with a suitable mixer.

    - Weight out 2 g of the homogenized sample and mix with 6 mL of ethyl acetate, vortex 3 min at max speed.

    - Centrifuge for 5 minutes at 4,000 x g at room temperature (20 – 25C / 68 – 77F).

    - Transfer 3 mL of the ethyl acetate supernatant (corresponding to 1 g of the original sample) into a new vial and use a rotary evaporator to dry the sample in a 60-70C water bath under reduced pressure. Alternatively, the sample can be dried by blowing nitrogen gas in a 60-70C water bath.

    - Dissolve the dried residue in 2 mL of n-hexane.

    - Add 1 mL of 1X Sample Extraction Buffer and mix by vortexing at maximum speed for 2 minutes.

    - Centrifuge for 10 minutes at 4,000 x g at room temperature. Discard the upper hexane layer. Take out 100 L of the lower aqueous layer, add 400 L of 1X Sample Extraction Buffer and mix well.

    - Use 100 L of the diluted sample per well for the assay.

Note: Dilution factor: 5.

### Fish/Shrimp/Meat (Beef/Chicken/Pork)

    - Homogenize a reasonable amount of sample with a suitable mixer.

    - Weight out 3 g of the homogenized sample and mix with 6 mL of ethyl acetate, vortex 3 min at max speed.

    - Centrifuge for 5 minutes at 4,000 x g at room temperature (20 – 25C / 68 – 77F).

    - Transfer 4 mL of the ethyl acetate supernatant (corresponding to 2 g of the original sample) into a new vial and use a rotary evaporator to dry the sample in a 60-70C water bath under reduced pressure. Alternatively, the sample can be dried by blowing nitrogen gas in a 60-70C water bath.

    - Dissolve the dried residue in 2 mL of n-hexane.

    - Add 1 mL of 1X Sample Extraction Buffer and mix by vortexing at maximum speed for 2 minutes.

    - Centrifuge for 10 minutes at 4,000 x g at room temp. Discard the upper hexane layer.

    - Use 100 L of the lower aqueous layer per well for the assay.

Note: Dilution factor: 0.5.

### Egg

- 1.   Homogenize a reasonable amount of sample with a suitable mixer.

- 2.   Weight out 2 g of the homogenized sample and mix with 10 mL of ethyl acetate, vortex 3 min at max speed.

- 3.   Centrifuge for 5 minutes at 4,000 x g at room temperature (20 – 25C / 68 – 77F).

- 4.   Transfer 5 mL of the ethyl acetate supernatant (corresponding to 1 g of the original sample) into a new vial and use a rotary evaporator to dry the sample in a 60-70C water bath under reduced pressure. Alternatively, the sample can be dried by blowing nitrogen gas in a 60-70C water bath..

- 5.   Dissolve the dried residue in 2 mL of n-hexane.

- 6.   Add 1 mL of 1X Sample Extraction Buffer and mix by vortexing at maximum speed for 2 minutes.

- 7.   Centrifuge for 10 minutes at 4,000 x g at room temperature. Discard the upper hexane layer.

- 8.   Use 100 L of the diluted sample per welI for the assay, in case emulsion happened, remove the upper n-hexane layer and incubate the lower aqueous layer in water bath for 3 min at 80-95°C

Note: Dilution factor: 1.

### Milk

    - For fat-free milk, dilute the milk sample with 1X Sample Extraction Buffer (1:4). (e.g. 50 L of milk + 200 L of buffer). Take 100 L of the diluted sample per well for the assay.

    - For the regular milk with fat, centrifuge the milk sample at 4,000 x g for 5 minutes, discard the upper fat layer. Dilute the samples with 1X Sample Extraction Buffer (1:4). (e.g. 50 L of milk + 200 L of buffer). Take 100 L of the diluted sample per well for the assay.

Note: Dilution factor: 5.

### Milk Powder

Organic Reagent Extraction Method

    - Weight out 2 g of milk powder in a centrifugal glass vial, add 10 mL of distilled water and dissolve by shaking.

    - Centrifuge at 4,000 x g for 5 minutes, discard the upper lipid layer.

    - Take 4 mL of milk from the aqueous layer, and add 6 mL of ethyl acetate, vortex for 3 minutes at maximum speed.

    - Centrifuge for 5 minutes at 4,000 x g at room temperature (20 – 25C / 68 – 77F).

    - Transfer 3 mL of the ethyl acetate supernatant (corresponding to 0.4 g of the original milk powder sample) into a new vial and use a rotary evaporator to dry the sample in a 60-70C water bath under reduced pressure. Alternatively, the sample can be dried by blowing nitrogen gas in a 60-70C water bath.

    - Dissolve the dried residue in 800 L 1X Sample Extraction Buffer.

    - Use 100 L per well for the assay.

Note: Dilution factor: 2.

Thiamphenicol ELISA TEST KIT PROTOCOL

### Reagent Preparation

IMPORTANT: All reagents should be brought up to room temperature before use (1 – 2 hours at 20 – 25C / 68 – 77F); Make sure you read “Warnings and Precautions” section on page 3. Solutions should be prepared just prior to ELISA test. All reagents should be mixed by gently inverting or swirling prior to use. Prepare volumes that are needed for the number of wells being run. Do not return the reagents to the original stock tubes/bottles. Using disposable reservoirs when handling reagents can minimize the risk of contamination and is recommended.

1.	Preparation of 1X Wash Solution

Mix 1 volume of the 20X Wash Solution with 19 volumes of distilled water.

### ELISA Testing Protocol

Label the individual strips that will be used and aliquot reagents as the following example:

| Component | Volume per Reaction | 24 Reactions |
| --- | --- | --- |
| HRP Conjugate | 50 L | 1.2 mL |
| 1X Wash Solution | 2.0 mL | 48 mL |
| Stop Buffer | 100 L | 2.4 mL |
| TMB Substrate | 100 L | 2.4 mL |

    - Add 100 L of each Thiamphenicol Standards in duplicate into different wells (Add standards to plate only in the order from low concentration to high concentration).

    - Add 100 L of each sample in duplicate into different sample wells.

    - Add 50 L of Thiamphenicol-HRP Conjugate and mix well by gently rocking the plate manually for 1 minute.

    - Incubate the plate for 1 hour at room temperature (20 – 25C / 68 – 77F) in the dark.

    - Wash the plate 3 times with 250 L of 1X Wash Solution. After the last wash, invert the plate and gently tap the plate dry on paper towels ( Perform the next step immediately after plate washings. Do not allow the plate to air dry between working steps).

    - Add 100 L of TMB substrate. Time the reaction immediately after adding the substrate. Mix the solution by gently rocking the plate manually for 1 minute while incubating. Incubate for 15-20 minutes at room temperature (20 – 25C / 68 – 77F) in the dark. (Do not put any substrate back to the original container to avoid any potential contamination. Any substrate solution exhibiting coloration is indicative of deterioration and should be discarded. Covering the microtiter plate while incubating is recommended).

    - After incubation, add 100 L of Stop Buffer to stop the enzyme reaction.

    - Read the plate as soon as possible following the addition of Stop Buffer on a plate reader with 450 nm wavelength (Before reading, use a lint-free wipe on the bottom of the plate to ensure no moisture or fingerprints interfere with the readings).

### Thiamphenicol Concentration Calculations

A standard curve can be constructed by plotting the mean relative absorbance (%) obtained from each reference standard against its concentration in ng/mL on a logarithmic curve.

Relative absorbance (%) =

Use the mean relative absorbance values for each sample to determine the corresponding concentration of the tested drug in ng/mL from the standard curve. A special program with Excel functionality,

The following figure is a typical Thiamphenicol standard curve. The sample detection and quantification limit for this kit are calculated as below.

Sample detection limit = (0.1 ng/g or ppb) x (dilution factor)

Sample quantification limit = (0.3 ng/g or ppb) x (dilution factor)

For example, the dilution factor for meat sample is 0.5, therefore, the detection limit for meat sample is 0.05 ng/g or ppb (0.1 ng/g x dilution factor 0.5) and the quantification limit is 0.15 ng/g or ppb (0.3 ng/g x dilution factor 0.5).

REAGEN LLC

Add.: 102CommerceDr.Unit8, Moorestown, NJ08057

Tel.:856-727-0250         FAX.:856-727-0251

E-mail:reagenkits@gmail.com  reagenllc@gmail.com

Web.: www.reagen.us