REAGEN™

Furazolidone (AOZ) ELISA Test Kit

RND99011

### Product Description

REAGEN™Furazolidone (AOZ) ELISA Test Kit provides a competitive enzyme mmunoassay for the quantitative analysis of furazolidone in fish, shrimp, meat (chicken、 pork and hepar), eggs, honey.

The unique features of the kit are:

- Rapid (4 hours), high recovery (75 - 105%), and cost-effective extraction methods for various samples.

- High sensitivity (0.02ng/g or ppb) and low detection limit (0.04 ng/g or ppb) for various samples.

- A quick ELISA assay (less than 2 hours regardless of number of samples).

- High reproducibility.

### Procedure Overview

The method is based on a competitive colorimetric ELISA assay. The drug of interest has been coated in the plate wells. During the analysis, sample and the AOZ antibody are added along with the secondary antibody, tagged with a peroxidase enzyme. If the AOZ residue is present in the sample, it will compete for the AOZ antibody, thereby preventing the antibody from binding to the AOZ-OVA attached to the well. The resulting color intensity, after addition of the HRP substrate (TMB), has an inverse relationship with the AOZ residue concentration in the sample.

### Kit Contents, Storage and Shelf Life

REAGEN™ Furazolidone (AOZ) ELISA Test Kit has the capacity for 96 determinations or testing of 42 samples in duplicate (assuming 12 wells for standards). Return any unused microwells to the foil bag and reseal them with the desiccant provided in the original package. Store the kit at 2-8C*. The shelf life is 12 months when the kit is properly stored.

| Kit Contents | Amount | Storage |
| --- | --- | --- |
| AOZ-coated Microtiter Plate | 96-well plate (8 wells x 12 strips) | 2-8C |
| AOZ Standards: Negative control (white cap tube) 0.02 ng/mL (yellow cap tube) 0.06 ng/mL (orange cap tube) 0.18 ng/mL (yellow cap tube) 0.54 ng/mL (pink cap tube) 1.62ng/mL (purple cap tube) 10 ng/mL (spiking, optional, red cap tube) | 1.0 mL 1.0 mL 1.0 mL 1.0 mL 1.0 mL 1.0 mL 1.0 mL | 2-8C |
| Antibody #1 | 6 mL | 2-8C |
| HRP-Conjugated Antibody #2 | 6 mL | 2-8C |
| 10X Sample Extraction Buffer | 25 mL | 2-8C |
| 20X Wash Solution | 30 mL | 2-8C |
| Stop Buffer | 12 mL | 2-8C |
| TMB Substrate | 12 mL | 2-8C |
| 50 mM 2-Nitrobenzaldehyde | 2.0 mL | 2-8C |

### Sensitivity

| Sample Type | Detection Limit (ng/g or ppb) |
| --- | --- |
| Shrimp/Fish/Meat/Hepar | 0.04 |
| Egg/Egg Powder | 0.04 |
| Honey | 0.04 |

### Specificity

| Analytes | Cross-Reactivity (%) |
| --- | --- |
| AOZ | 100 |
| AMOZ | <0.02 |
| AHD | < 0.01 |
| SEM | < 0.01 |

### Required Materials Not Provided With the Kit

- Microtiter plate reader (450 nm)

- Incubator

- Tissue Mixer (e.g. Omni TissueMaster Homogenizer)

- Rotary evaporator or nitrogen gas

- Vortex mixer (e.g. Gneie Vortex mixer from VWR)

- 10, 20, 100 and 1000 L pipettes

- Multi-channel pipette: 50-300 L (Optional)

- Ethyl acetate

- n-Hexane (or n-Heptane)

- 0.5M K2HPO4

- 1M NaOH

- 1M HCl

### Warnings and Precautions

- The standards contain Furazolidone . Handle with particular care.

- Do not use the kit past the expiration date.

- Do not intermix reagents from different kits or lots except for components with the same part No’s within their expiration dates. ANTIBODIES AND PLATES ARE KIT- AND LOT-SPECIFIC. Make sure that the antibody #2 and diluent are mixed in correct volumes.

- Try to maintain a laboratory temperature of 20°–25°C (68°–77°F). Avoid running assays under or near air vents, as this may cause excessive cooling, heating and/or evaporation. Also, do not run assays in direct sunlight, as this may cause excessive heat and evaporation. Cold bench tops should be avoided by placing several layers of paper towel or some other insulation material under the assay plates during incubation.

- Make sure you are using only distilled or deionized water since water quality is very important.

- When pipetting samples or reagents into an empty microtiter plate, place the pipette tips in the lower corner of the well, making contact with the plastic.

- Incubations of assay plates should be timed as precisely as possible. Be consistent when adding standards to the assay plate. Add your standards first and then your samples.

- Add standards to plate only in the order from low concentration to high concentration as this will minimize the risk of compromising the standard curve.

- Always refrigerate plates in sealed bags with a desiccant to maintain stability. Prevent condensation from forming on plates by allowing them equilibrate to room temperature (20 – 25C / 68 – 77F) while in the packaging.

SAMPLE PREPARATION

Be sure samples are properly stored. In general, samples should be refrigerated at 2-4°C for no more than 1-2 days. Freeze samples to a minimum of -20°C if they need to be stored for a longer period. Frozen samples can be thawed at room temps (20 – 25C / 68 – 77F) or in a refrigerator before use.

  - Preparation of 1X Sample Extraction Buffer:

Mix 1 volume of 10X Sample Extraction Buffer with 9 volumes of distilled water.

### Fish/Shrimp/Meat/Hepar

- Mix 1 g of the homogenized sample with 0.5 mL of 1X Sample Extraction Buffer, 3.5 mL of distilled water, 0.5 mL of 1 M HCl and 20 L of 50 mM 2-Nitrobenzaldehyde by vortexing for 30 seconds.

- Incubate at 50°C-55°C for 3 hours. Vortex the sample for 5 seconds every hour during the incubation.

- Add 5 mL of 0.1 M K2HPO4, 0.4 mL of 1 M NaOH and 6 mL of ethyl acetate, vortex for 1 minute.

- Centrifuge at 4,000 x g for 10 minutes at room temperature (20 – 25C / 68 – 77F).

- Transfer 3.0 mL of the ethyl acetate supernatant (corresponding to 0.5 g of the original sample) into a new vial ( Avoid the lower aqueous layer! If contaminated with lower layer, centrifuge the extracted ethyl acetate for 5 minutes at 4,000 x g again and get the upper organic layer). In case emulsion happened and the upper ethyl acetate layer was less than 3 mL, incubate the sample in water bath for 3 minutes at 85°C. Use a rotary evaporator to dry the sample in a 60-70C water bath under reduced pressure. Alternatively, the sample can be dried by blowing nitrogen gas in a 60-70C water bath.

- Dissolve the dried residue in 1 mL of n-hexane (or n-heptane).

- Add 1 mL of 1X Sample Extraction Buffer, vortex the sample for 2 minutes.

- Centrifuge the sample at 4,000 x g for 10 minutes at room temperature (20 – 25C / 68 – 77F).

- Use 50 L of the lower aqueous layer per well for the assay.

Note: Dilution factor: 2. To avoid high background, it is recommended that a solvent blank sample be prepared in parallel, starting with the reduction of 3 mL of ethyl acetate to dryness and continue with the rest extraction procedure. Subtract the result of the solvent control from the sample results. If the cut-off value is between 0.3 and 0.5 ppb, set the dilution factor at 6 by diluting 100 L of the lower aqueous layer in Step 9 with 200 L of 1 X Sample Extraction Buffer.

### Egg/Egg Powder

- Mix 1 g of the homogenized egg sample with 0.5 mL 1X Sample Extraction Buffer, 3.5 mL of distilled water, 0.5 mL of 1 M HCl and 20 L of 50 mM 2-Nitrobenzaldehyde by vortexing for 30 seconds.

- Incubate at 50°C-55°C for 3 hours. Vortex the sample for 5 seconds every hour during the incubation.

- Add 5 mL of 0.1 M K2HPO4, 0.4 mL of 1 M NaOH and 6 mL of ethyl acetate, vortex for 2 minutes.

- Centrifuge at 4,000 x g for 10 minutes at room temperature (20 – 25C / 68 – 77F).

- Transfer 3.0 mL of the ethyl acetate supernatant (corresponding to 0.5 g of the original egg sample) into a new vial ( Avoid the lower aqueous layer! If contaminated with lower layer, centrifuge the extracted ethyl acetate for 5 minutes at 4,000 x g again and get the upper organic layer). Use a rotary evaporator to dry the sample in a 60-70C water bath under reduced pressure. Alternatively, the sample can be dried by blowing nitrogen gas in a 60-70C water bath.

- Dissolve the dried residue in 1 mL of n-hexane (or n-heptane).

- Add 1 mL of 1X Sample Extraction Buffer and vortex the sample for 2 minutes.

- Centrifuge the sample at 4,000 x g for 10 minutes at room temperature (20 – 25C / 68 – 77F).

- Use 50 L of the lower aqueous layer per well for the assay.

Note: Dilution factor: 2. To avoid high background, it is recommended that a solvent blank sample be prepared in parallel, starting with the reduction of 3 mL of ethyl acetate to dryness and continue with the rest procedure. Subtract the result of the solvent control from the sample results. If the cut-off value is between 0.3 and 0.5 ppb, set the dilution factor at to 6 by diluting 100 L of the lower aqueous layer in Step 9 with 200 L of 1 X Sample Extraction Buffer.

### Honey

- Mix 1 g of the honey sample with 0.5 mL of 1X Sample Extraction Buffer, 3.5 mL of distilled water, 0.5 mL of 1 M HCl and 40 L of 50 mM 2-Nitrobenzaldehyde by vortexing for 1 minute

- Incubate at 50°C-55°C for 3 hours. Vortex the sample for 5 seconds every hour during the incubation.

- Add 5 mL of 0.1 M K2HPO4, 0.4 mL of 1 M NaOH and 6 mL of ethyl acetate, vortex for 2 minutes at max speed.

- Centrifuge at 4,000 x g for 10 minutes at room temperature (20 – 25C / 68 – 77F).

- Transfer 3.0 mL of the ethyl acetate supernatant (corresponding to 0.5 g of the original honey sample) into a new vial ( Avoid the lower aqueous layer! If contaminated with lower layer, centrifuge the extracted ethyl acetate for 5 minutes at 4,000 x g again and get the upper organic layer). Use a rotary evaporator to dry the sample in a 60-70C water bath under reduced pressure. Alternatively, the sample can be dried by blowing nitrogen gas in a 60-70C water bath.

- Dissolve the dried residue in 1 mL of n-hexane (or n-heptane).

- Add 1 mL of 1X Sample Extraction Buffer and vortex for 2 minutes.

- Centrifuge the sample at 4,000 x g for 10 minutes at room temperature (20 – 25C).

- Use 50 L of the lower aqueous layer for the assay.

Note: Dilution factor: 2. After evaporation of the ethyl acetate extract, the resulting residue is not completely dissolved. However, the recovery rate will not be compromised (>80%). If the cut-off value is between 0.3 and 0.5 ppb, set the dilution factor at 6 by diluting 100 L of the lower aqueous layer in Step 9 with 200 L of 1 X Sample Extraction Buffer.

**Alternative Step 1 and 2 (or step 2 and 3)

- Mix the suggested amount of sample (homogenize if necessary) with 4 mL of distilled water, 0.5 mL of 1 M HCl and 20 L of 50 mM 2-Nitrobenzaldehyde by vortexing for 30 seconds.

- Incubate at 37°C for 16 hours.

Furazolidone ELISA TEST KIT PROTOCOL

### Reagent Preparation

IMPORTANT: All reagents should be brought up to room temperature before use (1 – 2 hours at 20 – 25C / 68 – 77F); Make sure you read “Warnings and Precautions” section on page 3. Solutions should be prepared just prior to ELISA test.  All reagents should be mixed by gently inverting or swirling prior to use. Prepare volumes that are needed for the number of wells being run. Do not return the reagents to the original stock tubes/bottles. Using disposable reservoirs when handling reagents can minimize the risk of contamination and is recommended.

Preparation of 1X Wash Solution

Mix 1 volume of 20X Wash Buffer concentrate with 19 volumes of distilled water.

### ELISA Testing Protocol

Label the individual strips that will be used and aliquot reagents as the following example:

| Component | Volume per Reaction | 24 Reactions |
| --- | --- | --- |
| AOZ Antibody #1 | 50 L | 1.2 mL |
| 1X HRP-Conjugated Antibody #2 | 50 L | 1.2 mL |
| 1X Wash Solution | 2.0 mL | 48 mL |
| Stop Buffer | 100 L | 2.4mL |
| TMB Substrate | 100 L | 2.4 mL |

- Add 50 L of each AOZ Standards in duplicate into different wells ( Add standards to plate only in the order from low concentration to high concentration).

- Add 50 L of each sample in duplicate into different sample wells.

- Add 50 L of AOZ Antibody #1 and50 L of Antibody #2 solution ,then mix well by gently rocking the plate manually for 1 minute.

- Incubate the plate for 30 minutes at room temperature (20 – 25C / 68 – 77F). 

- Wash the plate 3 times with 250 L of 1X Wash Solution. After the last wash, invert the plate and gently tap the plate dry on paper towels ( Perform the next step immediately after plate washings. Do not allow the plate to air dry between working steps).

- Add 100 L of TMB substrate. Time the reaction immediately after adding the substrate. Mix the solution by gently rocking the plate manually for 1 minute while incubating ( F Do not put any substrate back to the original container to avoid any potential contamination. Any substrate solution exhibiting coloration is indicative of deterioration and should be discarded. Covering the microtiter plate while incubating is recommended).

After incubating for 15 minutes at room temperature (20 – 25C / 68 – 77F), add 100 L of Stop Buffer to stop the enzyme reaction.

REAGEN LLC

Add.:102CommerceDr.Unit8,Moorestown,NJ08057

Tel.:856-727-0251         FAX.:856-727-0250

E-mail:reagenkits@gmail.com  reagenllc@gmail.com

Web.: www.reagen.us

- Read the plate as soon as possible following the addition of Stop Buffer on a plate reader with 450nm nm wavelength ( Before reading, use a lint-free wipe on the bottom of the plate to ensure no moisture or fingerprints interfere with the readings).

### Furazolidone (AOZ) Concentration Calculations

A standard curve can be constructed by plotting the mean relative absorbance (%) obtained from each reference standard against its concentration in ng/mL on a logarithmic curve.

Relative absorbance (%) =

Use the mean relative absorbance values for each sample to determine the corresponding concentration of the tested drug in ng/mL from the standard curve.

The following figure is a typical furazolidone (AOZ) standard curve. The sample detection and quantification limit are calculated as below.

Sample detection limit = (0.05 ng/g or ppb) x (dilution factor)

Sample quantification limit = (0.15 ng/g or ppb) x (dilution factor)

For example, the dilution factor for meat sample is 2.0, therefore, the detection limit for meat sample is 0.1 ng/g or ppb (0.05 ng/g x dilution factor 2.0) and the quantification limit is 0.3 ng/g or ppb (0.15 ng/g x dilution factor 2.0).