REAGEN™

Benzalkonium chloride ELISA Test Kit

RND99065

### Product Description

REAGEN™ Benzalkonium chloride (BKC)ELISA Test Kit is a competitive enzyme immunoassay for the quantitative analysis of Benzalkonium chloride in various sample types.

The unique features of the kit are:

- Rapid extraction method for meat/tissue with high recovery (＞80%).

- High sensitivity (0.2 ng/g or ppb) .

- High reproducibility.

- A quick ELISA assay (less than 1 hours regardless of number of samples).

### Procedure Overview

The method is based on a competitive colorimetric ELISA assay. The BKC antibody has been coated in the plate wells. During the analysis, sample is added along with BKC-HRP. If the target is present in the sample, it will compete for the antibody, thereby preventing the  BKC-HRP from binding to the antibody attached to the well. The resulting color intensity, after addition of substrate, has an inverse relationship with the target concentration in the sample.

### Kit Contents, Storage and Shelf Life

REAGEN™ Benzalkonium chloride ELISA Test Kit has the capacity for 96 determinations or testing of 42 samples in duplicate (assuming 12 wells for standards). Return any unused microwells to the foil bag and reseal them with the desiccant provided in the original package. Most of the kit components should be stored at 2-8°C. The shelf life is 12 months when the kit is properly stored.

| Kit Contents | Amount | Storage |
| --- | --- | --- |
| BKC Ab-coated Microtiter Plate | 1 x 96-well plate (8 x 12 strips) | 2-8C |
| BKC Standards: Negative control (white cap tube) 0.2 ng/mL (yellow cap tube) 0.6 ng/mL (orange cap tube) 1.8 ng/mL (pink cap tube) 5.4 ng/mL (purple cap tube) 16.2 ng/mL (blue cap tube) 200 ng/mL (spiking, red cap tube) | 1.8mL 1.8mL 1.8mL 1.8mL 1.8mL 1.8mL 1.8mL | 2-8C |
| BKC HRP | 6mL | 2-8C |
| 20X Wash Solution ** | 30mL |  |
| TMB Substrate ** | 12mL |  |
| Stop Buffer ** | 14mL |  |
| 10X Sample Suspension Buffer | 10mL |  |

*	If you are not planning to use the kit for over 1 month, store HRP at -20C or in a freezer.

### Sensitivity

| Sample Type | Detection Limit (ng/g or ppb) |
| --- | --- |
| shrimp | 1 |

### Specificity

| Analytes | Cross-Reactivity (%) |
| --- | --- |
| Benzalkonium chloride | 100 |
| Polyaminopropyl biguanide | < 0.1 |
| Oxolin acid | < 0.01 |
| Nalidixic acid | < 0.01 |

### Required Materials Not Provided With the Kit

- Microtiter plate reader (450 nm)

- Incubator

- Tissue Mixer (e.g. Omni TissueMaster Homogenizer)

- Vortex mixer (e.g. Gneie Vortex mixer from VWR)

- 10, 20, 100 and 1000 L pipettes

- Multi-channel pipette: 50-300 L (Optional)

### Warnings and Precautions

- The standards contain Benzalkonium chloride. Handle with particular care.

- Do not use the kit past the expiration date.

- Do not intermix reagents from different kits or lots except for components with the same part No’s within their expiration dates. HRP AND PLATES ARE KIT-AND LOT-SPECIFIC.

- Try to maintain a laboratory temperature of 20–25°C (68–77°F). Avoid running assays under or near air vents, as this may cause excessive cooling, heating and/or evaporation. Also, do not run assays in direct sunlight, as this may cause excessive heat and evaporation. Cold bench tops should be avoided by placing several layers of paper towel or some other insulation material under the assay plates during incubation.

- Make sure you are using only distilled or deionized water since water quality is very important.

- When pipetting samples or reagents into an empty microtiter plate, place the pipette tips in the lower corner of the well, making contact with the plastic.

- Incubations of assay plates should be timed as precisely as possible. Be consistent when adding standards to the assay plate. Add your standards first and then your samples.

- Add standards to plate only in the order from low concentration to high concentration as this will minimize the risk of compromising the standard curve.

- Always refrigerate plates in sealed bags with a desiccant to maintain stability. Prevent condensation from forming on plates by allowing them equilibrate to room temperature (20 – 25C / 68 – 77F) while in the packaging.

SAMPLE PREPARATION

Be sure samples are properly stored. In general, samples should be refrigerated at 2-4°C for no more than 1-2 days. Freeze samples to a minimum of -20°C if they need to be stored for a longer period. Frozen samples can be thawed at room temperature (20–25C / 68–77F) or in a refrigerator before use. Preparation protocols for samples other than below can be made available upon request.

- Preparation of 1X BKC Sample Suspension Buffer

Mix 1 volume of 10X Sample Suspension Buffer with 2 volumes of 100% ethanol and 7 volumes of distilled water.

### Shrimp

- Homogenize the sample with a suitable mixer.

- Weigh out 1.0 g of the homogenized sample into a 15ml tube.

- Add 4ml of acetonitrile and 2ml of hexane.Vortex for 5 minutes at maximum speed or rotate on a multi-tube vortexer for 30 minutes.

- Centrifuge for 10 minutes at 4,000 x g at room temperature (20–25°C / 68–77°F).

- Remove the hexane layer. Transfer 0.5 mL of the clear supernatant to a tube, use a rotary evaporator to dry the sample in a 50-60°C water bath under reduced pressure. Alternatively, the sample can be dried by blowing nitrogen gas in a 50-60°C water bath.

- Add 0.5 mL of Sample Suspension Buffer and vortex vigorously for 1 minute.

- Use 100L for the assay.

Note: Dilution factor: 5

BKC ELISA TEST KIT PROTOCOL

### Reagent Preparation

IMPORTANT: All reagents should be brought up to room temperature before use (1 – 2 hours at 20 – 25C / 68 – 77F); Make sure you read “Warnings and Precautions” section on page 3. Solutions should be prepared just prior to ELISA test.  All reagents should be mixed by gently inverting or swirling prior to use. Prepare volumes that are needed for the number of wells being run. Do not return the reagents to the original stock tubes/bottles. Using disposable reservoirs when handling reagents can minimize the risk of contamination and is recommended.

- Preparation of 1X Wash Solution

Mix 1 volume of the 20X Wash Solution with 19 volumes of distilled water.

### ELISA Testing Protocol

Label the individual strips that will be used and aliquot reagents as the following example:

| Component | Volume per Reaction | 24 Reactions |
| --- | --- | --- |
| HRP-Conjugated | 50 L | 1.2 mL |
| 1X Wash Solution | 2.0 mL | 48 mL |
| Stop Buffer | 100 L | 2.4 mL |
| TMB Substrate | 100 L | 2.4 mL |

- Add 100 L of each Benzalkonium chloride Standards in duplicate into different wells ( Add standards to plate only in the order from low concentration to high concentration).

- Add 100 L of each sample in duplicate into different sample wells.

- Add 50 L of HRP-Conjugated. Incubate the plate for 30 minutes at room temperature (20–25°C / 68–77°F) (Avoid direct sunlight and cold bench tops during the incubation. Covering the microtiter plate while incubating is recommended).

Wash the plate 3 times with 250 L of 1 x Wash Solution. After the last wash, invert the plate and gently tap the plate dry on paper towels (Perform the next step immediately after plate washings. Do not allow the plate to air dry between working steps).

REAGEN LLC

Add.: 102CommerceDr.Unit8,Moorestown,NJ08057

Tel.:856-727-0250         FAX.:856-727-0251

E-mail:reagenkits@gmail.com    reagenllc@gmail.com

Web.:www.reagen.us

- Add 100 L of TMB substrate. Time the reaction immediately after adding the substrate. Mix the solution by gently rocking the plate manually for 1 minute while incubating ( Do not put any substrate back to the original container to avoid any potential contamination. Any substrate solution exhibiting coloration is indicative of deterioration and should be discarded. Covering the microtiter plate while incubating is recommended).

- After incubating for 20 minutes at room temperature (20 – 25°C / 68 – 77°F), add 100 L of Stop Buffer to stop the enzyme reaction.

- Read the plate as soon as possible following the addition of Stop Buffer on a plate reader with 450 nm wavelength ( Before reading, use a lint-free wipe on the bottom of the plate to ensure no moisture or fingerprints interfere with the readings).

### Benzalkonium chloride Concentration Calculations

A standard curve can be constructed by plotting the mean relative absorbance (%) obtained from each reference standard against its concentration in ng/mL on a logarithmic curve.

Relative absorbance (%) =

Use the mean relative absorbance values for each sample to determine the corresponding concentration of the tested drug in ng/mL from the standard curve.