REAGEN™

Colistin ELISA Test Kit

RND99046

### Product Description

REAGEN™ Colistin ELISA Test Kit is a competitive enzyme immunoassay for the quantitative analysis of Colistin in meat (beef, pork and chicken) and Egg.Colistin, also known as polymyxin E, is a cationic lipopeptide antibiotic isolated in 1950 from Bacillus polymyxa subsp. colistinus. It is most notable for its activity toward the multidrug-resistant organism Pseudomonas aeruginosa and low rates of resistance. Colistin is a mixture of cyclic polypeptides colistin A and B. Colistin is effective against most Gram-negative bacilli. The REAGEN™ Colistin ELISA Test Kit enables international and government regulatory agencies, food manufacturers and processors, as well as quality assurance organizations, to detect colistin in various sample types and to satisfy customer concerns about food safety

The unique features of the kit are:

- High recovery, rapid, and cost-effective extraction methods

- High sensitivity (0.5 ng/g or ppb)

- High reproducibility.

- A quick ELISA assay (less than 2 hours regardless of number of samples).

### Procedure Overview

The method is based on a competitive colorimetric ELISA assay. The drug of interest has been coated in the plate wells. During the analysis, sample is added along with the primary antibody specific for the target drug. If the target is present in the sample, it will compete for the antibody, thereby preventing the antibody from binding to the drug attached to the well. The secondary antibody, tagged with a peroxidase enzyme, targets the primary antibody that is complexed to the drug coated on the plate wells. The resulting color intensity, after addition of substrate, has an inverse relationship with the target concentration in the sample.

### Kit Contents, Storage and Shelf Life

REAGEN™ Colistin ELISA Test Kit has the capacity for 96 determinations or testing of 42 samples in duplicate (assuming 12 wells for standards). Return any unused microwells to the foil bag and reseal them with the desiccant provided in the original package. Store the kit at 2-8C *. The shelf life is 12 months when the kit is properly stored.

| Kit Contents | Amount | Storage |
| --- | --- | --- |
| Colistin-coated Microtiter Plate | 1 x 96-well plate (8 wells x 12 strips) | 2-8C |
| Standards: Negative control 0.5 ng/mL 1.5 ng/mL 5 ng/mL 15 ng/mL 50 ng/mL 1000 ng/mL (spiking, optional) | 0.8mL 0.8mL 0.8mL 0.8mL 0.8mL 0.8mL 0.8mL | 2-8C * |
| Colistin Antibody #1(powder) | 3 tubes | 2-8C |
| Antibody #1 Diluent | 14 mL |  |
| 100X HRP-Conjugated Antibody #2 | 250 L |  |
| Antibody #2 Diluent | 20 mL |  |
| 10X Colistin Extraction Buffer | 25 mL |  |
| 20X Wash Solution | 28mL |  |
| Stop Buffer | 14 mL |  |
| TMB Substrate | 10 mL |  |
| Clean Up Buffer I (Optional) Clean Up Buffer II (Optional) | 18 mL 18mL |  |

* 	If you are not planning to use the kit for over 1 month, store Colistin Standard , Colistin Antibody #1, 100X HRP-Conjugated Antibody #2 at -20C preferably or in a freezer.

### Sensitivity

| Sample Type | Detection Limit (ng/g or ppb) |
| --- | --- |
| Meat(Beef/Pork/Chicken) | 4 |
| Egg | 50 |

### Specificity

| Molecules | Cross-Reactivity (%) |
| --- | --- |
| Colistin | 100 |
| Polymyxin | 88 |
| Bacitracin zinc | < 0.5 |

### Required Materials Not Provided With the Kit

- Microtiter plate reader (450 nm)

- Incubator

- Tissue Mixer (e.g. Omni TissueMaster Homogenizer)

- Vortex mixer (e.g. Gneie Vortex mixer from VWR)

- 20, 100 and 1000 L pipettes

- Multi-channel pipette: 50-300 L (Optional)

- Acetonitrile，hexane

### Warnings and Precautions

- The standards contain colistin. Handle with particular care.

- Do not use the kit past the expiration date.

- Do not mix reagents from different kits or lots except for components with the same part No’s within their expiration dates. ANTIBODIES AND PLATES ARE KIT-AND LOT-SPECIFIC. Make sure that the antibody #2 and diluent are mixed in correct volumes.

- Try to maintain a laboratory temperature of 20°–25°C Avoid running assays under or near air vents, as this may cause excessive cooling, heating and/or evaporation. Also, do not run assays in direct sunlight, as this may cause excessive heat and evaporation. Cold bench tops should be avoided by placing several layers of paper towel or some other insulation material under the assay plates during incubation.

- Make sure you are using only distilled or deionized water since water quality is very important.

- When pipetting samples or reagents into an empty microtiter plate, place the pipette tips in the lower corner of the well, making contact with the plastic.

- Incubations of assay plates should be timed as precisely as possible. Be consistent when adding standards to the assay plate. Add your standards first and then your samples.

- Add standards to plate only in the order from low concentration to high concentration as this will minimize the risk of compromising the standard curve.

- Always refrigerate plates in sealed bags with a desiccant to maintain stability. Prevent condensation from forming on plates by allowing them equilibrate to room temperature (20 – 25C / 68 – 77F) while in the packaging.

SAMPLE PREPARATION

Be sure samples are properly stored. In general, samples should be refrigerated at 2-4°C for no more than 1-2 days. Freeze samples to a minimum of -20°C if they need to be stored for a longer period. Frozen samples can be thawed at room temperature (20 – 25C / 68 – 77F) or in a refrigerator before use.

- Preparation of 1X Colistin Extraction Buffer:

Mix 1 volume of 10X Colistin Extraction Buffer with 9 volumes of distilled water.

- Preparation of 20% Acetonitrile in 1X Colistin Extraction Buffer

Mix 1 volumes of acetonitrile with 4 volumes of 1X Colistin Extraction Buffer.

### Meat(beef, pork and chicken)

    - Remove fat from meat , homogenize a reasonable amount of sample with a suitable mixer.

    - Weigh out 1 g of the homogenized meat sample. Add 3 mL of 20% Acetonitrile in 1X Colistin Extraction Buffer and 200 uL of Clean Up Buffer I, vortex or shake for 10 seconds.

    - Add 200 uL of Clean Up Buffer II, vortex for 3 minutes manually at maximum speed or shake for 15 minutes at a multi-tube vortexer or shaker

    - Centrifuge for 10 minutes at 4,000 x g at room temperature (20 – 25C / 68 – 77F).

    - Use 75 uL of the suspension per well in the ELISA test.

Note: Dilution factor:4.4

### Egg

- Weigh out 1 g of the homogenized egg sample, add 4 mL of 1X Colistin Extraction Buffer.

- Vortex for 3 minutes manually at maximum speed or shake for 15 minutes at a multi-tube vortexer or shaker.

- Centrifuge for 10 minutes at 4,000 x g at room temperature (20 – 25C / 68 – 77F).

- Take 100 uL of the supernatant to a new tube, add 700 uL of 1X Colistin Extraction Buffer, 200 uL of acetonitrile, and 500 uL of hexane, vortex for 30 seconds.

- Centrifuge for 5 minutes at 4,000 x g at room temperature (20 – 25(C / 68 – 77(F), remove the top hexane layer completely.

- Use 75 uL of the bottom layer for the assay.

Note: Dilution factor: 50. .

COLISTIN ELISA TEST KIT PROTOCOL

### Reagent Preparation

IMPORTANT: All reagents should be brought up to room temperature before use (1 – 2 hours at 20 – 25C / 68 – 77F); Make sure you read “Warnings and Precautions” section on page 3. Solutions should be prepared just prior to ELISA test.  All reagents should be mixed by gently inverting or swirling prior to use. Prepare volumes that are needed for the number of wells being run. Do not return the reagents to the original stock tubes/bottles. Using disposable reservoirs when handling reagents can minimize the risk of contamination and is recommended.

- Preparation of 1X HRP-Conjugated Antibody #2

Mix 1 volume of 100X HRP-Conjugated Antibody #2 with 99 volumes of Antibody #2 Diluent.

- Preparation of 1X Wash Solution

Mix 1 volume of the 20X Wash Solution with 19 volumes of distilled water

- Preparation of Antibody#1 Solution

Based on the needed amount of antibody, take out one or more Colistin Antibody#1 tubes (powder), add 4 mL of Antibody#1 Diluent to each tube, invert the tube up and down for 10 times, leave the tube at room temperature for at least 30 min, invert the tube up and down for another 10 times. The antibody solution is ready to use. The reconstituted antibody is stable for 1 week at 4C or 1 month at -20C freezer.

### ELISA Testing Protocol

Label the individual strips that will be used and aliquot reagents as the following example:

| Component | Volume per Reaction | 24 Reactions |
| --- | --- | --- |
| Colistin Antibody #1 | 100 L | 2.4 mL |
| 1X-HRP Conjugate Antibody #2 | 150 L | 3.6 mL |
| 1X Wash Solution | 2.0 mL | 48 mL |
| Stop Buffer | 100 L | 2.4 mL |
| TMB Substrate | 100 L | 2.4 mL |

- Add 75 L of each Colistin Standard in duplicate into different wells ( Add standards to plate only in the order from low concentration to high concentration).

- Add 75 L of each sample in duplicate into different sample wells.

- Add 100 L of Colistin Antibody #1 to each well and mix well by gently rocking the plate manually for 1 minute.

- Incubate the plate for 50 minutes at room temperature (20 – 25C / 68 – 77F).

- Wash the plate 3 times with 250 L of 1X Wash Solution. After the last wash, invert the plate and gently tap the plate dry on paper towels (Perform the next step immediately after plate washings. Do not allow the plate to air dry between working steps).

- Add 150 L of 1X HRP-Conjugated Antibody #2. Incubate the plate for 20 minutes at room temperature (20 – 25C / 68 – 77F) (Avoid direct sunlight and cold bench tops during the incubation. Covering the microtiter plate while incubating is recommended).

- Wash the plate 3 times with 250 L of 1X Wash Solution. After the last wash, invert the plate and gently tap the plate dry on paper towels (Perform the next step immediately after plate washings. Do not allow the plate to air dry between working steps).

- Add 100 L of TMB substrate. Time the reaction immediately after adding the substrate. Mix the solution by gently rocking the plate manually for 1 minute while incubating (Do not put any substrate back to the original container to avoid any potential contamination. Any substrate solution exhibiting coloration is indicative of deterioration and should be discarded. Covering the microtiter plate while incubating is recommended).

- After incubating for 15 minutes at room temperature (20 – 25C / 68 – 77F), add 100 L of Stop Buffer to stop the enzyme reaction.

- Read the plate as soon as possible following the addition of Stop Buffer on a plate reader with 450 nm wavelength ( Before reading, use a lint-free wipe on the bottom of the plate to ensure no moisture or fingerprints interfere with the readings).

### Colistin Concentration Calculations

A standard curve can be constructed by plotting the mean relative absorbance (%) obtained from each reference standard against its concentration in ng/mL on a logarithmic curve.

Relative absorbance (%) =

Use the mean relative absorbance values for each sample to determine the corresponding concentration of the tested drug in ng/mL from the standard curve.

The following figure is a typical Colistin standard curve.

REAGEN LLC

Add.: 102CommerceDr.Unit8,Moorestown,NJ08057

Tel.:856-727-0250         FAX.:856-727-0251

E-mail:reagenkits@gmail.com  reagenllc@gmail.com

Web.:www.reagen.us