REAGEN™

Saxitoxin (PSP) ELISA Test Kit

RNA97009

### Product Description

REAGEN™ Saxitoxin (PSP) ELISA Test Kit is a competitive enzyme immunoassay for the quantitative analysis of Saxitoxin in mussels and water samples. Saxitoxin, the principal component of paralytic shellfish poisons (PSP), blocks the sodium ion channels in nerve and muscle membranes.The lethal dose in humans is 1 to 3 mg. Numbness and respiratory paralysis can occur after oral ingestion of as little as 0.5 - 1.0g saxitoxin.

Saxitoxin residues can be determined by either GC-MS or HPLC. However, enzyme immunoassays (ELISA) show considerable advantages as it is faster and more sensitive.

REAGEN™ Saxitoxin (PSP) ELISA Test Kit enables international and government regulatory agencies, food manufacturers and processors, as well as quality assurance organizations, to detect saxitoxin in shellfish matrices and to satisfy customer concerns about food safety.

The unique features of the kit are:

- High recovery (80%-105%), rapid (10 - 30 minutes), and cost-effective extraction methods

- High sensitivity (0.05 ng/g or ppb) and low detection limit (3 ppb for mussel samples and 1 ppb for water samples).

- High reproducibility.

- A quick ELISA assay(less than 1.5 hours regardless of number of samples).

### Procedure Overview

The method is based on a competitive colorimetric ELISA assay. The second antibody against anti-saxitoxin antibody was coated in the plate wells. During the analysis, sample is added along with the saxitoxin-horseradish peroxidase (Saxitoxin-HRP) conjugate and anti-saxitoxin antibody. If the saxitoxin residue is present in the sample, it will compete with Saxitoxin-HRP for the anti-saxitoxin antibody, thereby preventing the Saxitoxin-HRP from binding to the anti-Saxitoxin antibody. The anti-saxitoxin antibodies are then bound by the second antibody immobilized on the plate. After a washing step and addition of the TMB substrate solution, a color signal is produced. The color reaction is stopped after a specified time and the color is evaluated using an ELISA reader. The resulting color intensity has an inverse relationship with the saxitoxin residue concentration in the sample.

### Kit Contents, Storage and Shelf Life

REAGEN™ Saxitoxin (PSP)  ELISA Test Kit has the capacity for 96 determinations or testing of 42 samples in duplicate (assuming 12 wells for standards). Return any unused microwells to the foil bag and reseal them with the desiccant provided in the original package. Store the kit at 2-8C *. The shelf life is 12 months when the kit is properly stored.

| If you are not planning to use the kit for over 1 months, store Anti-Saxitoxin Antibody and HRP-Conjugated at -20C or in a freezer. Kit Contents | Amount | Storage |
| --- | --- | --- |
| Second antibody-Coated Plate | 1 x 96-well Plate(8 wells x 12 strips) | 2-8C |
| Saxitoxin (PSP) Standards: Negative control (white cap tube) 0.05 ng/mL (yellow cap tube) 0.15 ng/mL (orange cap tube) 0.5 ng/mL (pink cap tube) 1.5ng/mL (purple cap tube) 4.5ng/mL (blue cap tube) | 0.8 mL 0.8 mL 0.8 mL 0.8 mL 0.8mL 0.8 mL | -20C |
| Anti-Saxitoxin Antibody | 6 ml | 2-8C |
| Saxitoxin-HRP Conjugated | 6 mL |  |
| 20X Wash Solution | 28 mL |  |
| Stop Buffer | 14mL |  |
| TMB Substrate | 12 mL |  |
| 10X Sample Extraction Buffer | 25 mL |  |

### Sensitivity

| Sample Type | Detection Limit (ng/g or ppb) |
| --- | --- |
| Mussels | 3 |
| Water | 1 |

### Specificity

| Analytes | Cross-Reactivity (%) |
| --- | --- |
| Saxitoxin | 100 |
| Neosaxitoxin | 20 |
| Decarbamoyl Saxitoxin | 100 |
| GTX 2 & 3 | 43 |
| GTX 1 & 4 | 2 |
| Decarbamoyl GTX 2 & 3 | 10 |
| Decarbamoyl Neosaxitoxin | 4 |
| GTX-5 | 61 |

### Required Materials Not Provided With the Kit

- Microtiter plate reader (450 nm)

- Incubator

- Tissue Mixer

- Vortex mixer

- 10, 20, 100 and 1000 L pipettes

- Multi-channel pipette: 50-300 L (Optional)

- Methanol

### Warnings and Precautions

- The standards contain a small amount of Saxitoxin . Handle with particular care.

- Do not use the kit past the expiration date.

- Do not intermix reagents from different kits or lots except for components with the same part No’s within their expiration dates.

- Try to maintain a laboratory temperature of 20°–25°C (68°–77°F). Avoid running assays under or near air vents, as this may cause excessive cooling, heating and/or evaporation. Also, do not run assays in direct sunlight, as this may cause excessive heat and evaporation. Cold bench tops should be avoided by placing several layers of paper towel or some other insulation material under the assay plates during incubation.

- Make sure you are using distilled or deionized water since water quality is important.

- When pipetting samples or reagents into an empty microtiter plate, place the pipette tips in the lower corner of the well, making contact with the plastic.

- Incubations of assay plates should be timed as precisely as possible. Be consistent when adding standards to the assay plate. Add your standards first and then your samples.

- Add standards to plate only in the order from low concentration to high concentration as this will minimize the risk of compromising the standard curve.

- Always refrigerate plates in sealed bags with a desiccant to maintain stability. Prevent condensation from forming on plates by allowing them equilibrate to room temperature (20 – 25C / 68 – 77F) while in the packaging.

SAMPLE PREPARATION

Be sure samples are properly stored. In general, samples should be refrigerated at 2-4°C for no more than 1-2 days. Freeze samples to a minimum of -20°C if they need to be stored for a longer period. Frozen samples can be thawed at room temps (20 – 25C / 68 – 77F) or in a refrigerator before use.

- Preparation of 1x Sample Extraction Buffer:

- Mix 1 volume of 10X Sample Extraction Buffer with 9 volumes of distilled water

- Preparation of 1x Sample Extraction Buffer/Methanol(90/10, V/V)：

- Mix 9 volumes of 1X Sample Extraction Buffer with 1 volume of methanol.

Note: The extracted sample can be further diluted using 1X Sample Extraction Buffer for higher level of determination.

### Mussels

- Remove the mussel tissue samples from shells, wash with deionized water.

- Drain the excess liquid and homogenize the sample to a soupy texture.

- Weigh out 0.5 g of homogenized tissue, add 1 mL of 50% methanol(in water) and vortex for 3 minutes at maximum speed.

- Centrifuge tube for 10 minutes at 4,000 rpm.

- Transfer 0.1 mL clear supernatant to a new tube.

- Add 1.9 mL of 1X Sample Extraction Buffer/Methanol (90/10, V/V), mix well.

- Using 50L of the sample for the assay.

Note: Dilution factor: 60.

Alternative Sample Preparation

- Remove the mussel tissue samples from shells, wash with deionized water.

- Drain the excess liquid and homogenize the sample to a soupy texture.

- Weigh out 0.5 g of homogenized mussels, add 2 mL of 1X Sample Extraction Buffer. Vortex for 3 minutes at maximum speed.

- Centrifuge 10 minutes at 4000 rpm.

- Transfer 0.1 mL of clear supernatant to a new tube.

- Add 1.1 mL of 1X Sample Extraction Buffer/Methanol (90/10, V/V), mix well.

- Using 50L of the sample for the assay

- Note: Dilution factor: 60.

### Water

- Centrifuge the water sample at 4,000 rpm for 5 minutes. Transfer 100 L of the sample clear supernatant to a new tube.

- Add 1.9 mL of 1X Sample Extraction Buffer/Methanol (90/10, V/V), mix well.

- Take 50 L of the diluted sample per well for the assay.

Note: Dilution factor: 20.

SAXITOXIN (PSP) ELISA TEST KIT PROTOCOL

### Reagent Preparation

IMPORTANT: All reagents should be brought up to room temperature before use (1 – 2 hours at 20 – 25C / 68 – 77F); Make sure you read “Warnings and Precautions”

section on page 2. Solutions should be prepared just prior to ELISA test. All reagents should be mixed by gently inverting or swirling prior to use. Prepare volumes that are needed for the number of wells being run. Do not return the reagents to the original stock tubes/bottles. Using disposable reservoirs when handling reagents can minimize the risk of contamination and is recommended.

- Preparation of 1X Wash Solution

Mix 1 volume of 20X Wash Buffer concentrate with 19 volumes of distilled water.

### ELISA Testing Protocol

Label the individual strips that will be used and aliquot reagents as the following example:

| Component | Volume per Reaction | 24 Reactions |
| --- | --- | --- |
| Anti-Saxitoxin Antibody | 50L | 1.2mL |
| Saxitoxin-HRP-Conjugated | 50L | 1.2mL |
| 1X Wash Solution | 1.0 mL | 24mL |
| Stop Buffer | 100 L | 2.4 mL |
| TMB Substrate | 100 L | 2.4 mL |

- Add 50L of each Saxitoxin Standards in duplicate into different wells ( Add standards to plate only in the order from low concentration to high concentration).

- Add 50L of each sample in duplicate into different sample wells.

- Add 50L of saxitoxin-HRP Conjugated to each well

- Add 50L of Anti-Saxitoxin Antibody to each well and mix well by gently rocking the plate manually for 1 minute.

- Incubate the plate for 30 minutes at room temperature (20 – 25°C / 68 – 77°F).

- Wash the plate 3 times with 250L of 1X Wash Solution. After the last wash, invert the plate and gently tap the plate dry on paper towels ( Perform the next step immediately after plate washings. Do not allow the plate to air dry between working steps).

- Add 100L of TMB substrate. Time the reaction immediately after adding the substrate. Mix the solution by gently rocking the plate manually for 1 minute while incubating. ( Do not put any substrate back to the original container to avoid any potential contamination. Any substrate solution exhibiting coloration is indicative of deterioration and should be discarded. Covering the microtiter plate while incubating is recommended).

- After incubation for 15 minutes in room temperature, add 100L of Stop Buffer to stop the enzyme reaction.

- Read the plate as soon as possible following the addition of Stop Buffer on a plate reader with 450 nm wavelength ( Before reading, use a lint-free wipe on the bottom of the plate to ensure no moisture or fingerprints interfere with the readings).

### Saxitoxin (PSP) Concentration Calculations

A standard curve can be constructed by plotting the mean relative absorbance (%) obtained from each reference standard against its concentration in ng/mL on a logarithmic curve.

Relative absorbance (%) =

The following figure is a typical Saxitoxin (PSP) standard curve

REAGEN LLC

Add.: 102CommerceDr.Unit8,Moorestown,NJ08057

Tel.:856-671-1789         FAX.:856-858-5434

E-mail:reagenkits@gmail.com  reagenllc@gmail.com

Web.: www.reagen.us